GPO Box 1321

Katherine NT 0850

ABN:4746956090

REQUEST FOR TENDER/QUOTE

| PROJECT NUMBER: |  |
| --- | --- |
| RFT/Q NUMBER: | RGRC-PM-2023/02 |
| RFT/Q TITLE: | Bulman dump road upgrade |
| CONTACT PERSON FOR MORE INFORMATION: | Dave heron projects@ropergulf.nt.gov.au 08-89729018 |
| CLOSING DATE FOR SUBMISSIONS: | 5pm 30 april 2023 (NB: late tenders/quotes will not be accepted) |

| - LODGEMENT OF TENDERS/QUOTES TO: |  |
| --- | --- |
| The Tender and Quote Box or via Local Buy Vendor Panel |  |
| By Electronic Lodgement: projects@ropergulf.nt.gov.au |  |
| By Hand: | By Post: |
| Roper Gulf Regional Council 29 Crawford St Katherine NT 0850 | PO Box 1321 Katherine NT 0851 |
| By Local Buy https://www.localbuy.net.au/ |  |

TABLE OF CONTENTS

1.	Project specific requirements	3

2.	response schedules	10

- PREPARED BY:

Roper Gulf Regional Council Governance, Corporate Planning & Compliance

When preparing your submission you are advised to refer to the General Contract Conditions Annexure, which detail general requirements applicable to this Request for Tender/Quote (RFT/Q). The General Contract Conditions Annexure is available at https://ropergulf.nt.gov.au/system/files/uploads/files/2022/General%20Contract%20Conditions%20Annexure.pdf

Hard copies of the Annexure are available at the Roper Gulf Regional Council Commercial Operations administration office at 2 Crawford St Katherine.

PROJECT SPECIFIC REQUIREMENTS

This Tender/Quote is subject to the General Contract Conditions Annexure.

|  |  |  |  | RFT/Q NO | RGRC-PM-2023/02 |
| --- | --- | --- | --- | --- | --- |
| 1. | For further information contact | Name | Dave Heron |  |  |
|  |  | Telephone | 08-89729018 |  |  |
|  |  | Facsimile |  |  |  |
|  |  | Email | projects@ropergulf.nt.gov.au |  |  |
| 2. | Contact for Site Inspection |  | Annette Kelly, CSC Bulman |  |  |
|  | Contact Details |  | 08-8975 4189 |  |  |
|  | Location of Briefing |  | Bulman NT |  |  |
|  | Designated Time/Day for Inspection /Meeting |  | On request |  |  |

| 3. |  | Subject to Industry Accreditation |  | N/A |  |
| --- | --- | --- | --- | --- | --- |
| 4. |  | Alternative Quotes |  | No |  |
| 5. |  | Part Offer Acceptable |  | No |  |
| 6. | Quote Validity Period |  |  | 60 days, Changes are not allowed |  |
|  | (Response Schedule) |  |  |  |  |
| 7. |  | Documents to be lodged with Quote |  |  |  |
| QUOTE FORM DECLARATION OF BUSINESS STATUS LUMP SUM PRICE BREAK DOWN ANY OTHERS AS REQUESTED |  |  |  |  |  |
| 8. |  |  | Indigenous Development Plan Required | Yes |  |
| Optimising Indigenous Opportunities- What services suppliers and sub-contractors will use on this project to optimise indigenous employment outcomes and community benefit? |  |  |  |  |  |

| 9. | Specific Site Conditions apply | No |  |
| --- | --- | --- | --- |
| 10. | Specific Sub-Contractors apply |  | No |
|  | (Clause 3.9 – General Contract Conditions Annexure) |  |  |
| 11. | Basis of Payment |  | By invoice 14 days terms |
|  | (Clause 4.14– General Contract Conditions Annexure) |  |  |

| Continued…. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RFT/Q NO |  |  |  |
| 12. | Project Control Plan required | No |  |  |  |  |
| 13. | Public Liability Insurance required Work Cover Other insurances (if required) Professional Indemnity Product Liability | PLI not less than $10,000,000.00 |  |  |  |  |
|  | (Clause 3.6 – General Contract Conditions Annexure) |  |  |  |  |  |
| 14. | Anticipated commencement date | May 2023 |  |  |  |  |
| 15. | Anticipated completion date | July 2023 |  |  |  |  |
| 18. | Retention Sum | 5% of Contract Price |  |  |  |  |
|  | (Clause 4.14.7 – General Contract Conditions Annexure) |  |  |  |  |  |
| 19. | Project Manager | Title |  | Mr Dave Heron |  |  |
|  |  | Address |  | 63 Chardon Street Katherine |  |  |
|  |  | Contact |  | 08-89729018 |  |  |
| 20. | Safety Management Plan Required | Yes |  |  |  |  |
|  | (Clause 1.2 – General Contract Conditions Annexure) |  |  |  |  |  |
| 21. | Mandatory Site Inspection | Yes |  |  |  |  |
|  | (Clause 4.19 – General Contract Conditions Annexure) |  |  |  |  |  |

Project Specific Requirements are outlined below and are to be used in conjunction with the General Contract Conditions Annexure

The selection of specific items or materials for the works being carried out is specified in these Project specific Requirements or shown in the annexure/s.

You are advised to ensure you are familiar with the rules of Quoting and with the contractual obligations of the parties under any subsequent contract. You are further advised to reference the General Contract Conditions Annexure, which appear via this link

https://ropergulf.nt.gov.au/system/files/uploads/files/2022/General%20Contract%20Conditions%20Annexure.pdf which detail specific requirements applicable to the RFT/Q.

Precedence

Any provision in the project specific requirements remains consistent and refers back to the relevant clause and section in General Contract Conditions Annexure.

ASSESSMENT CRITERIA

Quotation proposals will be evaluated on the following criteria

- Demonstrated previous experience – include details of works of similar previous projects, experience of working in the Northern Territory or other remote locations (30%);

- Appropriately qualified staff to install equipment at locations – please outline portfolios/resumes of staff you propose (10%);

- Capacity to complete projects within requested timeframes (10%);

- Commitment to safety (10%);

- Costs – costs are reasonably competitive and are acceptable within Councils budget constraints (30%).

- Indigenous Engagement – commitment to engage the local people in the project. (10%)

- Companies may provide information on any relevant factors addressing the selection criteria, which may assist the Principal in making an assessment of the quotation.

Assessments will also be based on:

- Completeness (basic contractual / commercial compliance);

- Conformity with specified requirements;

- Comparative evaluation based on achieving best value for money;

- Quality of product / service;

- Compliance with Sub-contractor / supplier provisions;

- Local factors eg. extent of support for Northern Territory business and industry;

- Company's technical, financial and managerial capacity;

- Company's willingness to engage Aboriginal and/or Torres Strait Islander people when delivering Services;

- Company's ability to demonstrate an understanding of the issues affecting Aboriginal and/or Torres Strait Islander people; and

- Company's ability to communicate sensitively and effectively with Aboriginal and/or Torres Strait Islander people.

- Company's level of supervision and safe work practices.

- Company’s ability to produce high quality documents.

## Scope of WORK or Specification

# SCOPE OF WORKS

The works consist of forming an unsealed road to upgrade the dump road at Bulman, from the end of the existing sealed road to the dump site.

The scope of Work comprises, but is not limited to the following:

- Rip, form, and compaction of subbase for 1.1 kms by 7 meters wide formation from end of seal in town to dump boundary.

- Make allowance for 2 cement stabilised floodways to allow for drainage.

- Place a concrete floodway across the creek 20 lm long and 4 meters wide and150 mm thick. Allow for an edge footing 200mm below bottom of slab around all sides of slab. Allow for SL82 reo and 25 mpa concrete.

- On the section coming out of a creek up a rocky hill at the dump end of job, reshape the corner around the hill to stop erosion.

- Make allowance to bring in any fill required.

- Supply and placement of fill in various locations along roadway to form a road and fill depressions along existing track before placing base course.

- Supply, placement, and compaction of screened gravel road base for 1.1 kms at 150mm thick by 6 meters wide.

- Do a dial before you dig for locations of any services.

- Locate, select, operate and rehabilitate the gravel source as per requirements in any MEA supplied by The Roper gulf council.

Include in the quote:

- All materials, including road base and fill for low areas.

- Supply 25 mpa Concrete and SL82 reinforcing.

- Placement and finishing of concrete.

- All machinery to complete the job.

- Ongoings including accommodation.

Not included in the quote:

- Any NLC fees or charges for the use of gravel from any pits, this will be the council’s responsibility.

- Any council fees or charges

- It is the Roper Gulf council’s responsibility to obtain all relevant permits required to source gravel and fill.

## site of works

SPECIFIC SITE CONDITIONS

# water and gravel resources

Construction water is permitted to be extracted from the following locations:

|  | Resource | Latitude | Longitude | Chainage |
| --- | --- | --- | --- | --- |
| Water Point 1 | Construction water | -13.657318° | 134.362436° | 259.8 km |
| Water Point 2 Wilton River | Construction water | -13.652263° | 134.369058° | 260.8 km |

It’s the contractor’s responsibility to locate, select, operate and rehabilitate the gravel source.

# SITE RULES - WORK ON COMMUNITIES

- Contractors are advised that restrictions may apply to entering and working in an Aboriginal Community.

- It is the Contractor's responsibility to ascertain from the relevant Community Council or Land Council details of any permits conditions, restrictions, requirements, fees etc. applicable to working in that Community.

- All permissions, permits and charges are the responsibility of the Contractor.

- A Volatile Substance Abuse Management Plan may apply in and/or near the area of the works. Information can be found at https://health.nt.gov.au/professionals/alcohol-and-other-drugs-health-professionals/volatile-substances

# DIAL BEFORE YOU DIG

- Dial Before You Dig (DBYD) is a community service that provides information on underground pipes and cables from all underground service providers in the area.  There is no cost to the user for this service; however, some asset owners reserve the right to make a modest charge for some plans.

- Contact DBYD at least 2 working days before you plan to excavate and obtain the plans of the underground services before commencing excavation.

- Provide a written notice to the Superintendent stating that the plans have been received and include the DBYD sequence number.

- DBYD can be contacted at the following numbers:

- Telephone:	1100	8am to 5pm weekdays, excluding public holidays

- Internet:	www.1100.com.au

# LIGHT AND POWER

- The Contractor shall install any temporary electric light and power installations, including metering, required for executing the work under the Contract all in accordance with the relevant regulations, and shall pay all costs and charges in connection with the installation and use of such services.

# Water

- The Contractor shall arrange for the supply of all water required for executing the Works under the Contract and pay all incidental charges, including cost of water so used.

# SANITARY ACCOMMODATION

- The Contractor shall provide adequate and clean sanitary accommodation for use of all persons employed upon the Works and clear away and remove such accommodation from the site on completion of the Contract.

# Camp Site/Compound/Workshop – Hold point

- Hold Point - Provide a copy of written permission from the owner or lessee of the land permitting establishment and operation of a camp site, compound and/or workshop, before commencing works.

- Pay all costs associated with the use of the site(s).

- Refer to the Department of Health - Environment Health Fact Sheet No.700 for Camp Site Requirements for Mining and Construction Projects.

- Maintain all facilities in good condition.

- Maintain buildings in relation to fire protection in accordance with the Northern Territory Building Act 1993 and Northern Territory Fire and Emergency Act 1996 and associated Regulations.

- Please note there is no available accommodation in Bulman.

## Food Preparation Facilities – Hold Point

- Hold Point - Provide a copy of proof of registration with Department of Health of any camp food preparation area in accordance with the Food Act.

## Waste

- Comply with the requirements of the Waste Management and Pollution Control Act 1998.

- Remove from the site and dispose of all waste materials, including green waste, food scraps and other putrescible wastes, construction waste, chemicals and effluent in an appropriate manner, in approved legal waste disposal sites or facilities.

- Failure to comply with these requirements may result in remedial action being taken at your cost and may result in legal action being taken.

## Effluent – Hold Point

- Hold Point - Provide a copy of written approval from Department of Health (DoH) for any proposed on-site effluent disposal system, before commencing works.

- Ensure that all effluent from amenities is discharged into an approved facility or, if permitted by the controlling authority, the local sewerage system. Effluent disposal direct to ground or water is NOT permitted.

- Septic tanks and portable self-contained toilets of suitable capacity may be used subject to suitable arrangements for the disposal of effluent.

- Hold Point - Where the use of septic tanks or portable toilets is not reasonable or practical, pit toilets may be used, but this requires the prior written approval of the Superintendent.

- Any pit toilets constructed must be at least 100 m from any bore, at least 200 m from any watercourse and sites must be appropriately rehabilitated on completion.

- All septic tank installations or alternative septic systems servicing buildings both within and outside of declared building control areas, apart from installations subject to the Building Act 1993, must be approved by the Chief Health Officer (CHO) or the CHO's delegate for the area in which the works are to be carried out. Further information may be obtained from the relevant Environmental Health Officer in whose area the works are to be located. Regional contacts are contained within the Code of Practice for On-Site Waste Water Management accessible via https://nt.gov.au/property/building-and-development/wastewater-management/codes-and-guidelines.

## Rehabilitation – Hold Point

- Hold Point - Obtain approval from the Superintendent for the completed rehabilitation of the camp site/compound/workshop before final demobilisation.

- On completion of the works remove all facilities, unless otherwise agreed in writing with the owner or lessee of the land and restore the site to a clean and tidy condition.

- Rehabilitate the site to its condition prior to conducting site works for establishing the camp ground, compound and/or workshop unless another course of action is approved by the Superintendent.

- Where the camp site/compound /workshop is located within an extraction area, rehabilitation is to be undertaken in accordance with the conditions applying to rehabilitation of extraction areas. Rehabilitation measures must be implemented to a standard approved by the Superintendent prior to final demobilisation. Refer to the Standard Specification for Environmental Management.

# EXTRACTION AREAS And Water Sources

- Take responsibility for locating, selecting, operating, and rehabilitating all borrow pits and water sources.

- Roper Gulf council will be responsible for any permits and fees associated for the use of gravel pits.

- No town water is available for construction purposes.

## Crushing or Screening – Witness Point

- The crushing or screening plant to be used on the project subject to this contract must be certified as fit for use by a competent person. The certification of fitness for use must have been issued not more than one year prior to the date of the scheduled completion of the works plus one calendar month. A competent person is defined in the NT Work Safe Bulletin 09.01.16 Competent Persons for Inspection and Maintenance of Plant.

- Comply with the guidance provided in the Safe Work Australia Code of Practice Managing Risks of Plant in the Workplace.

## Operation of Extraction Areas – Hold Point

- Access

- Construct only one access road to each pit.

- Confine all transport operations to the access road.

- Provide and maintain adequate road drainage

.

- Extraction

- Strip 100 mm minimum depth top layer throughout the area of operation.

- Stockpile stripped material clear of drainage courses to a maximum height of 2 m.

- Ensure that side slopes of sand or gravel are not steeper than one vertical to two horizontal at any time when the excavation is unattended.

- Remove or bury by-products of the excavation operations unless otherwise required.

- Limit of Excavation

- Not within 6 m of any fence line or utility service.

- Not within any gas pipeline reserve.

- Not within sight of road traffic.

- Not within 125 m of any road or railway centre line.

- Not within 25 m of a water course.

- Maximum area: Should not exceed 1 ha.  Align the long side with the contour.

- Hold Point - Obtain Superintendent approval to exceed 1ha pit size.

- Maximum width: 50 m.

- Maximum depth: 2 m.

- Leave natural vegetation strips 25 m width between pits.

- Stockpile cleared vegetation and subsequently spread over the surface of the extraction area.

- Existing pits within 125 m of a public road may be used provided:

- No significant revegetation exists.

- Extension proceeds away from the road.

- Site is rehabilitated after use.

## Rehabilitation of Extraction Areas

- Progressively rehabilitate extraction areas.

- Backfill all test pits.

- Respread unused material and rip 0.5 m deep at 3 m spacing along the contours.

- Remove all rubbish and debris.

- Replace stockpiled topsoil and cleared vegetation uniformly over the extraction area.

- Batter walls at three horizontal to one vertical where excavation is less than 1 m depth, and six horizontal to one vertical where depth exceeds 1 m.

- Rehabilitate any access road constructed for the project.

- Refer to the Standard Specification for Environmental Management.

## Stream Sites

- Contact Department of Environment and Natural Resources (DENR) prior to conducting any work in a stream site.

- Excavation Limits

- Not within 200 m upstream or downstream of any road structure, pipeline or gauging station.

- Not in a manner liable to cause erosion or further disturbance to the watercourse.

- Not within 15 m of the trunk of a tree and not under the branches of any tree.

- Conditions

- Leave sizeable islands to ensure groupings of trees that will withstand stream bed erosion.

- Maximum batter slope: Two horizontal to one vertical.

# Natural Gravel

Obtain material from sources of naturally occurring deposits.

Produce required properties by crushing, screening, blending, mixing or other processes necessary.

Ensure particles are tough, durable and of a tightly binding nature free of organic or other deleterious matter.

| Table – Natural Gravel Particle Sizes |  |  |  |  |
| --- | --- | --- | --- | --- |
| AS Sieve (mm) | Percentage Passing |  |  |  |
|  | Type 1 | Type 2 | Type 3 | Type 4 |
| 75.0 | 100 |  |  | 100 |
| 37.5 | 80 - 100 | 100 |  | 80 - 100 |
| 19.0 | 50 - 80 | 70 - 100 | 100 | 60 - 100 |
| 9.5 | 35 - 65 | 50 - 80 | 70 - 100 | 50 - 95 |
| 4.75 | 25 - 50 | 35 - 65 | 50 - 80 | 40 - 80 |
| 2.36 | 15 - 40 | 25 - 50 | 35 - 65 | 30 - 65 |
| 0.425 | 7 - 20 | 10 - 30 | 15 - 35 | 20 - 50 |
| 0.075 | 3 - 13 | 4 - 16 | 6 - 20 | 5 - 25 |

Compaction is to be at 95% MMDD for formation and gravel base course.

All base course materials are to be screened gravel Type 3.

# CONCRETE

| Table – Concrete properties |  |
| --- | --- |
| Property | Required measure |
| Compressive strength | N25 |
| Aggregate size, generally | 20 mm |
| Aggregate size, machine extruded kerbs and gutters | 10 mm |
| Slump, generally | 80 mm, + or - 15 mm |
| Slump, machine extruded kerbs and gutters | 40 mm |

| Table – Concrete Finishes |  |  |
| --- | --- | --- |
| Type | Description | Application |
| S1 | Left rough to give key but not honeycombed or porous | Surfaces to be rendered. |
| S2 | Wood float | As specified. |
| S3 | Steel trowel without polish | Internal surfaces subject to foot traffic. Kerb and gutter. |
| S4 | Wood float and broomed finish - broom finish - broom across direction of traffic | Surfaces subject to vehicular traffic. |
| S6 | Steel float followed by moist hair broom | Surfaces subject to foot traffic. |
| F1 | Remove mortar fins, etc., repair minor blow holes by bagging where approved or rub down with Carborundum stone | Formed surfaces exposed to view. |
| F2 | Off forms | - |
| F3 | Exposed RCG | Application of RCG to be hand spread once application of the exposed mix has been bull floated. RCG to be measured 1000 grams per square meter, or as otherwise specified by the Superintendent. Colour and size of RCG to be specified by Superintendent. |

TENDER/QUOTE RESPONSE SCHEDULE – Contractor to complete

| RFT/Q Number |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RFT/Q Title |  |  |  |  |  |  |  |  |
| HOW TO RESPOND TO THIS TENDER/QUOTE |  |  |  |  |  |  |  |  |
| Complete the quote form and schedules provided. This will become your Quote which may be lodged in any of the following ways: |  |  |  |  |  |  |  |  |
| By Electronic Lodgement facility at: projects@ropergulf.nt.gov.au |  |  |  | By Hand Delivery to: Roper Gulf Regional Council Office 2 Crawford St Katherine NT 0850 |  |  |  |  |
|  |  |  |  | By Post: Po Box 1321 Katherine NT 0851 |  |  |  |  |
| By Local Buy Vendor Panel |  |  |  | https://www.localbuy.net.au/ |  |  |  |  |
| Quotes should remain valid for acceptance for a period of 60 days from the closing date. |  |  |  |  |  |  |  |  |
| THIS SECTION TO BE COMPLETED BY THE RESPONDENT |  |  |  |  |  |  |  |  |
| I/We, the undersigned, having examined and acquired an actual knowledge of this Request for Tender/Quote do hereby offer to perform the whole of the Works in accordance with this Request for Tender/Quote in the amount of |  |  |  |  |  |  |  |  |
| Amount Quoted | $ |  |  | Including GST |  |  |  |  |
| I herby declare I am quoting for whole of the scope of work |  |  |  |  |  |  |  |  |
| Or part of the scope of work which include; |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  | Date |  |  |
| (print name and date) |  |  |  |  |  |  |  |  |
| If applicable, I/We confirm receipt and inclusion in the Tender/Quote, addenda numbered: |  |  |  |  |  |  |  |  |
| Legal Entity |  |  |  |  |  |  |  |  |
| (Name of legal entity – include trading name) |  |  |  |  |  |  |  |  |
| ACN/BN |  |  |  |  | CAL Registration Number |  |  |  |
| ABN |  |  |  |  |  |  |  |  |
| Postal Address |  |  |  |  |  |  |  |  |
| Telephone |  |  |  |  |  |  |  |  |
| e-mail address |  |  |  |  |  |  |  |  |

| CONTACT PERSON DETAILS |  |  |  |
| --- | --- | --- | --- |
| Name |  | Position |  |
| Telephone |  |  |  |
| e-mail address |  |  |  |

| DECLARATION OF BUSINESS STATUS (Select the appropriate entity type and complete details) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sole Trader | Full Name of Proprietor |  |  |  |  |
|  | Business Name (if applicable) |  |  |  |  |
|  Partnership | Names of All Partners |  |  |  |  |
|  | Partnership Name (if applicable) |  |  |  |  |
|  Company | Company Name |  |  |  |  |
|  | Business Name (if different from Company name) |  |  |  |  |
|  | Names of all Directors |  |  |  |  |
|  | Names of Holding and Subsidiary Companies (if applicable) |  |  |  |  |
|  | ACN/BN of Holding and Subsidiary Companies |  |  |  |  |
|  Trust | Copy of Trust Deed Attached |  | Yes |  |  |
|  |  |  | No |  |  |
|  Joint Venture | Name of all Parties |  |  |  |  |
|  | ACN/BN of all Parties |  |  |  |  |
| CERTIFICATION I certify on behalf of __________________________________________(the Quoter), To the best of my knowledge: - None of the Proprietors, Directors, Managers is bankrupt or a Director, Manager or Secretary of a Company that is being wound up (whether voluntary or otherwise), and - The business is not trading under: - An arrangement and / or reconstruction (i.e. restructuring a public company) - Receiver and management - Official management - An arrangement with creditors without sequestration (i.e. with the proprietors being made bankrupt). |  |  |  |  |  |
| Signed |  |  |  | Dated |  |
| For |  |  |  |  |  |
|  | (Name of legal entity) |  |  |  |  |

| LUMP SUM PRICE BREAKDOWN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Complete the mandatory Lump Sum Price Breakdown Schedule by inserting the prices for each individual part of the work as set out below. All prices, where applicable, must be inclusive of GST. This Lump Sum Price Breakdown schedule is required for quote assessment purposes an will be used as a basis for progress payment and either a Plus or Minus variation will be created when a final measurement of installed material. |  |  |  |  |
| DESCRIPTION |  | Unit Cost |  | Amount (GST Inclusive) |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
|  |  | $ |  | $ |
| Miscellaneous |  | $ |  | $ |
| LUMP SUM AMOUNT – Total must match the amount shown on the QUOTE FORM |  | $ |  |  |
| Signed |  |  | Dated |  |
| For |  |  |  |  |
|  | (Name of legal entity) |  |  |  |